Standard Conditions of Approval – Aerobic Treatment Units

THE FOLLOWING CONDITIONS OF APPROVAL SHALL BE FULLY COMPLIED WITH BEFORE A PERMIT TO USE APPARATUS TO ALLOW OCCUPATION OF THE HOME WILL BE  ISSUED.

- This approval is for the installation of a _________________________ Aerobic Treatment Unit with an effluent disposal system of the following category:

| - Spray irrigation | - Sub-strata irrigation | - Sub-surface irrigation |
| --- | --- | --- |

(The setback distances indicated overleaf for the irrigation system type must be complied with).

- The System shall comply with the Code of Practice for the Design, Manufacture and Operation of ATUs Serving Single Dwellings and all conditions of approval imposed by the Executive Director, Public Health.

- The ATU shall be positioned 1.2 metres from any boundary, building or structure and 6 metres from wells, bores or any water course.

- An approved audible and visual alarm device is required on the pump system and shall be installed within the home.

- The use of food waste disposal units is not permitted with Aerobic Treatment Units

- One spa bath up to maximum capacity of 350 litres may be connected to an ATU, Spa baths more than 350 litres must be connected to a separate disposal system. Spa pools are not permitted to be connected to an ATU.

- The effluent disposal area is to be positioned by the District Environmental Health Officer in conjunction with the applicant and property owner before the site works commence. Any additional conditions that may be imposed on the application must be complied with.

- All storm water and subsoil drainage (where provided) shall be discharged away from the effluent disposal area.

- Unless otherwise approved, the effluent disposal area shall have an area of 150 square metres with a gradient of less than 1:20 (dripper irrigation) / 1:10 (spray irrigation). The effluent disposal area shall not be split into more than two separate areas.

- Where the irrigation area contains areas of laterite cap rock the rock must be fracture to a depth of 600mm below the waste water contact level.

- Where dripper irrigation is used each zone must be installed with a self-flushing valve attached to the lowest point of the collection manifold and a vacuum breaker valve at the highest point in system.

- A minimum free area of 100 square metres is required (additional to the effluent disposal system, all structures, carports, trafficable areas, access paving and setbacks) (not required for subsurface drippers).

- At least two signs are to be maintained in the effluent disposal area warning that the water is unsuitable for drinking. The effluent disposal area is to be planted with suitable plants for the surface irrigation disposal area (copy attached) within 30 days of occupancy of the premises.

- The effluent disposal area requires a minimum or 300mm of clean, friable, permeable, clay-free soil below the waste water contact level, retained by an impervious bund. A minimum of 50mm of mulch (spray irrigation) / 100mm of mulch (substrata drippers) / 150mm of soil cover (subsurface drippers) shall be maintained.

- All plumbing before the ATU shall be undertaken by a licensed plumber.

- All electrical works shall be undertaken by a licensed electrician in accordance with AS/NZS 3000.

- An Installation Compliance Certificate shall be submitted upon completion.

- An owner’s manual shall be provided to the property owner(s) upon completion.

- NOTE: THE CONDITIONS WHICH HAVE BEEN APPLIED TO YOUR APPLICATION HAVE RESULTED FROM A SITE ASSESSMENT AND MAY BE ALTERED BY THE COUNCIL IF SITE WORKS RELATING TO THE INSTALLATION REVEAL CONDITIONS WHICH COULD NOT BE OBSERVED AT THE TIME OF THE INITIAL SITE INSPECTION.

Environmental Health Officer

# The Effluent Disposal Area Shall Comply With The Following Minimum Setbacks:

| ## Item | ## Spray irrigation | ## Sub-strata irrigation | ## Sub-surface irrigation | ## Aquarius Aquasafe Drains |
| --- | --- | --- | --- | --- |
| ## Open Fenced Boundaries | ## 1.8 metres | ## 0.5 metres | ## 0.5 metres | ## 1.8 metres |
| ## Closed Fenced Boundaries ## a minimum of 1.2m high | ## As determined by the local government | ## 0.5 metres | ## 0.5 metres | ## 1.8 metres |
| ## Buildings | 1.8 metres ** | ## 0.5 metres | ## 0.5 metres | 1.8 metres |
| ## Sub-soil/ Open drains | ## 6.0 metres | ## 3.0 metres | ## 3.0 metres | ## 6.0 metres |
| ## Swimming Pools | ## 3.0 metres *** | ## 2.0 metres | ## 2.0 metres | 1.8 metres |
| ## Bores/Wells/Creeks | ## 30.0 metres | ## 30.0 metres | ## 30.0 metres | ## 30.0 metres |
| ## ATUs | ## 1.8 metres | ## 0.5 metres | ## 0.5 metres | ## 1.8 metres |
| ## Paths, driveways, carports and other structures | ## 1.8 metres | ## 0.5 metres | ## 0.5 metres | ## 1.8 metres |

**	Where the site has a severe slope and a building (including those on adjoining lots) is downhill from the effluent disposal area, this setback is altered to 3.0 metres.

***Where the site has a severe slope and a swimming pool (including those on adjoining lots) is downhill from the effluent disposal area, this setback is altered to 6.0 metres.