Asbestos Containing Materials (ACM) Complaints -

Site Investigation

Complaint ID 	__________________      File Ref __________________

Date 		__________________

Officer		______________________________

Sketch lot inc. major features

Site address  ____________________________

_______________________________________

Contact Person___________________________

Position ________________________________

Email __________________________________  Phone _________________________________

Postal address___________________________

Indicate sample location and size on plan

1. ______________

2. ______________

3. ______________

4. _______________

5. _______________

6. _______________

Description of Contamination

____________________________________________________________________________________________________________________________________________________________

□ Photographs taken

Walkover Results – select representative area

Grid A (indicate location on plan)

Size (eg: 3m x 3m) _____________ = ____m2

Piece sizes located in grid (cm2): _________________________________________________________

Total = ________cm2. Divide by total area (m2) = _______cm2/m2.

Risk Rating = 		□ Very Low (<10cm2/m2 with no soil disturbance)

□ Low – (> 10cm2 and/or significant soil disturbance and/or buried fence stumps)

□ Med - High (large quantities and/or soil disturbance).

Notes and Onsite Discussions Include names & positions. Detail all activities which are to be halted.

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

_____________________________________________________________________________________

Required Immediate Actions – Site Owner

(tick all that are required)

- Isolation of area

  - Fencing

  - Securing facility (if indoors)

  - Other ____________________________

- Prevention of dust

  - Hydro-mulching

  - Wind fencing

  - Wetting of soil

  - Site drainage (to prevent water erosion)

  - Other ____________________________

- Notification of neighbours/adjacent occupiers

- Signage at entry/exit points (see over page)

Required Actions – EHO/LG

(tick all that are required)

- Contact other bodies – required where site is non-residential or multi residential or not “very low” risk.

  - DoH Toxicology 9388 4984

  - DoH Applied EH 9388 4965

  - Worksafe

  - DEC

  - Other__________________

- Issue cleanup / removal notice

(R8. Health (Asbestos) Regs 1992)

- Notify as suspected contaminated site (Form 1 – Contaminated Sites Act 2003)

- Other ____________________________

_________________________________

Supporting Information – ACM Investigations

General Information

Though some Officers may identify Asbestos Containing Materials (ACM) from experience it is recommended to have the findings verified by a laboratory. If in doubt assume the material is ACM.

Possible Cleanup Actions

Note – Clean up action should not be undertaken in adverse weather conditions. Ie: windy etc.

Very Low Risk

- Wet soil to depth of 10cm.

- Rake with fine rake to uncover buried ACM fragments.

- Remove all fragments.

- EHO to verify. Tipping receipts to be provided.

Low Risk

- Consult DoH with expectation that excavation of soil will be required.

- Tipping receipts to be provided.

Med - High Risk

- Consult DoH

- Expectation that sampling/ remediation and verification will be required.

- Inform owner of legal responsibility of reporting to DEC under Contaminated Sites Act.

- LG to consider reporting of owner is unwilling.

Minimum Removal Equipment

General

- Gloves

- P1/P2 dust mask

- 0.2mm thick plastic sufficient to double wrap material to be disposed of.

- Rake

- Water to wet soil.

Excavations

- Site to be sprayed with wetting agent

- Secure site, remove all people not involved in cleanup

- Erect warning signs (see below)

- Inform neighbours

- Cover soil for transport

Warning Sign Example Wording

CAUTION

CONTAINS ASBESTOS FIBRE
AVOID CREATING DUST
SERIOUS INHALATION HEALTH HAZARD

CAUTION
RESPIRATORY PROTECTION MUST BE WORN
NO ADMITTANCE - ASBESTOS
REPORT TO SITE/PROPERTY MANAGER

References

- Guidelines for the Assessment, Remediation and Management of Asbestos Contaminated Sites in Western Australia. May 2009.  Appendix B and C. Department of Health WA.

- Management of asbestos in the non-occupational environment. enHealth. 2005.