Applicant ___________________________  App#___________

Lot____ No._____ Street ______________________________

Suburb _______________________________

EHO ____________________________ Date _____________

Initial Site Inspection - ATU Applications

| # | Setbacks | / |
| --- | --- | --- |
| 1 | - Irrigation - 30m to wells, bores & creeks. - Tank – 6m to wells, bores, dam or creek. |  |
| 2 | 3m to subsoil drains, soak-wells & drainages easements. |  |
| 3 | 0.5m to highest known water table (0.65m for subsurface irrigation), (1.8m for reduced irrigation areas in sand). |  |
| 4 | Fencing near boundary open or closed? |  |
| 5 | Tanks at least 1.2m from structures and boundaries |  |
| 6 | Irrigation area meets required setbacks – see table ## Item; ## Spray irrigation; ## Sub-strata irrigation; ## Sub-surface irrigation; ## Aquarius Aquasafe Drains. ## Open Fenced Boundaries; ## 1.8m; ## 0.5m; ## 0.5m; ## 1.8m. ## Closed Fenced Boundaries (min 1.2m high); ## As set by the LGA; ## 0.5m; ## 0.5m; ## 1.8m. ## Buildings; 1.8m **; ## 0.5m; ## 0.5m; 1.8m. ## Sub-soil/ Open drains; ## 6.0m; ## 3.0m; ## 3.0m; ## 6.0m. ## Swimming Pools; ## 3.0m***; ## 2.0m; ## 2.0m; 1.8m. ## ATUs; ## 1.8m; ## 0.5m; ## 0.5m; ## 1.8m. ## Paths, driveways, carports and other structures; ## 1.8m; ## 0.5m; ## 0.5m; ## 1.8m. ** 3m for severe slope and a building (including those on adjoining lots) is downhill. ***6m for severe slope and a swimming pool (including those on adjoining lots) is downhill. |  |
| 7 | 100m2 area spare for social recreation? (not req. for subsurface irrigation) |  |
|  | Land Surface |  |
| 8a | Gradient less than 1:20 – all irrigation types OK. |  |
| 8b | Gradient between 1:20 and 1:10 –spray irrigation ok without terracing, dripper irrigation to be terraced – cutoff drains req. |  |
| 8c | Gradient between 1:10 and 1:5 – irrigation area to be terraced for all irrigation types. |  |
| 8d | Gradient – over 1:5 – unsuitable for irrigation |  |
| 9 | At least 150m2 without rock outcrops |  |
| 10 | Not a valley floor or low lying area subject to inundation? |  |
| 11 | If shown on plan, is stormwater discharged away from disposal area? |  |
| 12 | No evidence of perched water table? |  |
|  | Soil Type & System Size |  |
| 13 | Depth to impervious layer (bedrock) at least 300mm? If no: actual depth? |  |
| 14 | Description of soil type: |  |
| 15 | Appropriately sized system? (Reduced irrigation area available for sandy soils – refer to ATU COP). |  |
| # | Comments |  |