Applicant ___________________________  App#___________

Lot____ No._____ Street ______________________________

Suburb _______________________________

EHO ____________________________ Date _____________

Initial Site Inspection – Grey water Applications

| # | Setbacks | / |
| --- | --- | --- |
| 1 | - Irrigation – 30m to wells, bores & creeks. - Tank – 6m to wells, bores, dam or creek. |  |
| 2 | 6m to subsoil drains, soak-wells & drainages easements. |  |
| 3 | Depth to highest known water table - 0.65m subsurface irrigation, 0.9m for trenches |  |
| 4 | Fencing near boundary open or closed? |  |
| 5 | Tanks at least 1.2m from structures and boundaries |  |
| 6 | Irrigation area meets required setbacks – see table ## Item; ## Subsurface Dripper irrigation; ## Sub soil trench. ## Open Fenced Boundaries; ## 0.5m; ## 1.2m. ## Closed Fenced Boundaries (min 1.2m high); ## 0.3m; ## 0.5m. ## Buildings; ## 0.5m; 1.2m. ## Sub-soil/ Open drains; ## 3.0m; ## 6.0m. ## Swimming Pools; ## 2.0m; 3.0m. ## Paths, driveways, carports and other structures; ## 0.3m; ## 0.5m. Note – spray and sub strata dripper irrigation only allowed if secondary treatment of effluent has occurred (ie: ATU type operation) |  |
| 7 | Overflow to main system/sewer is/is not required? |  |
|  | Land Surface |  |
| 8 | Gradient - less than 1:10 – all irrigation types OK. - between 1:10 and 1:5 – terracing required - over 1:5 – unsuitable for irrigation |  |
| 9 | At least 150m2 without rock outcrops |  |
| 10 | Not a valley floor or low lying area subject to inundation? |  |
| 11 | If shown on plan, is stormwater discharged away from disposal area? |  |
| 12 | No evidence of perched water table? |  |
|  | Soil Type & System Size |  |
| 13 | Depth to impervious layer (bedrock) at least 300mm? If no: actual depth? |  |
| 14 | Description of soil type: |  |
| 15 | Appropriately sized system? |  |
| 16 | Cut off Drain is/is not required? |  |
| 17 | Pump tank is/is not required (min. 250L with overflow, 250L+1 days retention without overflow) |  |
| # | Comments |  |