Applicant ___________________________  App#___________

Lot____ No._____ Street ______________________________

Suburb _______________________________

EHO ____________________________ Date _____________

Initial Calculations & Site Inspection

Effluent Disposal Applications

Septic Tanks and Leach Drains

| # | Setbacks |  | / |
| --- | --- | --- | --- |
| 1 | 30m to wells, bores & creeks. |  |  |
| 2 | 6m to subsoil drains, soak-wells & drainages easements. |  |  |
| 3 | 1.2m to highest known water table |  |  |
| 4 | Drains at least 1.8m from structures and boundaries |  |  |
| 5 | Tanks at least 1.2m from structures and boundaries |  |  |
| 6 | Minimum 1.8m between drains, unless otherwise approved |  |  |
| 7 | Drains run parallel with land contours |  |  |
|  | Land Surface |  |  |
| 9 | Gradient less than 1 in 5 |  |  |
| 10 | At least 150m2 without rock outcrops (excludes Laterite) |  |  |
| 11 | Not a valley floor or low lying area subject to inundation? |  |  |
| 12 | If shown on plan, is stormwater discharged away from disposal area? |  |  |
| 13 | No evidence of perched water table? |  |  |
|  | Soil Type |  |  |
| 14 | Depth to impervious layer (bedrock) at least 1.2m? If no: actual depth? |  |  |
| 15 | Description of soil type: |  |  |
| 16 | Soil absorption test performed? (Schedule G of Regs.) Time for water to fall 25mm? ______ Soil Type according to Regs? _____________________________ |  |  |
|  | Calculations |  |  |
| 17 | Appropriately sized system? (See Sizing Septic Tanks & Leach Drains sheet) |  |  |
| 18 | Adequate fall to drains & Septics? |  |  |
|  | LEACH DRAINS Ground level at leach drain ______m Minus Lid and Invert Level - 0.30m Plus fall: longest run/80 (_____/80) +_____m Plus head allowance + 0.60m Total (Req’d Finished Floor Level) =______m | SEPTIC TANKS Finished Floor Level ______m Plus Lid and Invert Level + 0.30m Minus fall: longest run/80 (_____/80) -_____m Minus head allowance - 0.60m Req’d Level of Tanks = _______m If Rq’d level of Tanks is < GL at tanks on plan -Risers are Required If Rq’d level of Tanks is > GL at tanks on plan - Risers are not Required |  |
| # | Comments |  |  |